Automatic Packaging Machinery Co., Ltd

Model:220

Manual

Contents

One.  Description of equipment                                P2
1. summarize                                             P2

2. performance and specification                              P2

Two .   The machine configuration diagram                       P3

Three.  Installation instructions                                 P3

Four.  Man-machine interface debugging method                   P5

Five.  Equipment adjustment                                   P9

1. Each bag weighs adjust error                               P9

2. Give material time adjust                                  P10

3. Former and vertical heating adjustment                      P10

4.  Bag’s unilateral adjustment                              P11

5. Sponge inflatable adjustment                              P11

6. External air source inflatable adjustment                     P11

7. The vertical sealing flanging adjustment                     P11

Six. Common fault reasons and exclusion                         P12

Seven. Equipment maintenance                                 P13

Eight. The circuit principle diagram(one)                        P14

Nine.  The circuit principle diagram(two)                        P15

Ten.  220 Pneumatic chart                                    P16

Eleven.  With machine accessories list                 P17

One ,  Description of equipment

1. summarize

220 Automatic packaging system is our factory introducing foreign advanced technology and digestion ,absorption and production of a kind of high quality and high performance full-automatic packaging machine. The machine can finish automatically all processes, from measuring, filling, bag making , dage stamp and finished goods conveyance. The machine adopts high accuracy servo film system and PLC program operation control, advanced touch human-machine interface, photoelectric tracking and digital temperature automatic positioning, so as to make the operation simple, perfect, Let the food packaging industry improve efficiency and reduce labor intensity, improving packing grade is the preferred equipment

2. performance and specification

| Power | Single-phase 220V/50HZ |  | Total power | 2.2KW |
| --- | --- | --- | --- | --- |
| Packing material | OPP/CPP、CPP/PE MST/PE、PET/PE |  | - horizontal seal heat power( 2 pieces series) | 800W/220VAC |
| Packing thickness | 0．05-0.08mm |  | - Vertical heat power(2 pieces series) | 500W/220VAC |
| Max. Film width | 420mm |  | Measuring range | 150-1000mL |
| Max bag forming dimension | 220mm(W)x300mm(L) |  | Using pressure | 0.65MPa |
| Packing speed | 5-60 bags/min |  | Air consumption | 600L/min |
| Machine weight | 350kg |  | Dimension | 1030(L)x930(W)x1250(H) |
| Application | Puffed food, Cookies, beans, peanuts, melon seeds, tea and flake, poedered |  |  |  |

Remark: Packaging materials used volumes, outer diameter :≤400mm  skeleton inner diameter Φ 75mm, packing materials for 0.05-0.08 mm thickness, smooth, no wavy ,edge within the scope of 10mm not bearing decorative pattern, color words, etc. (namely, be pure with the background color punctuation) must contrast significantly.

Two .   The machine configuration diagram

Three  Installation instructions

1. packaging machine, finished product conveyer, access with the leakage protection belt access over 10A, 220VAC power

- Automatic elevator access with leakage, phase over 10A power three-phase 380VAC power .

3. Please don't forget to install grounding

4. Access the gas supply to the packaging machine three conjoined at the entrance

5. Use elevator transport the material to the material hopper

6. Date of installation printing ribbon

7. Packaging film installation

1） Low scraper, remove plate cores

2） Load packaging film

3） Load plate cores, handle

4） Tighten the screw, scraper over by living packaging film, remove the handle

5） The thick line direction in a continuous line and will send packaging film into former. (we shall first be vertical sealing open). Looking following map

6) Let the relaxation of C segment to block the photoelectric switch reflector screen

7) After packaging film enter into former, adjust swinging rod down through A place to packaging film forming device collar into material barrel

8) A and B point ring of shaft stick at the edge of the packaging film

9) Let block the photoelectric switch squeegee tilted phase contact with packaging film

Now at the bottom of the packaging machine photoelectric switch and  film motor into the automatic working state

Four.  Man-machine interface debugging method

1.Turn on the power, and the screen display

homepage

Welcome to use Foshan Soonk Packaging machine

Welcome to use Automatic Packaging machine

中文   English

Chinese

English

2.Touch “English” screen display:(figure)

| Production operation | Debug operation | Other functions |
| --- | --- | --- |
| Combination weigher | Cup measuring device | Auger filler |
| Parameter Settings | Alarm Settings | Back to language |

3. Need to set each function parameters, touch this function  as follows:

- Touch “Production operation ”screen display:

| Production operation bag/units output Color standard Open Close Temperature control Open Close |  |  |  |  |
| --- | --- | --- | --- | --- |
| Debug operation | Adjustments | Other functions | Combination weigher | Back to homepage |

| horizontal seal roller turn one time vertical seal roller turn one time cutter to cut one time Load film one time inflatable one time Continuous action again Continuous debuging operation Open Close |  |
| --- | --- |
| Production operation Other functions Combination weigher Cup measuring device |  |
| Auger filler Parameter Settings Alarm Settings | Back to homepage |

- Return to the main menu, machine operation menu options

Touch “Debug operation” ,screen display

First touch “manual”： “manual” become dark

Touch “horizontal seal ”：means the horizontal seal roller turn one time.

Touch “Vertical seal” : means vertical seal roller turn one time.

Touch “cutter”: means the cutter will cut in setting time.

Touch “Continuous action again”: means each function will according to set parameters to operat one time.

Finished a empty bag operation( this time,the turntable not operate,means not give material)

| Double load film Open Close Insert Angle Open Close Inflatable Open Close Lingking bag Open Close |  |
| --- | --- |
| Production operation Debuging operation Combination weigher Cup measuring device |  |
| Auger filler Parameter Setting Alarm Settings | Back to homepage |

- Return to the main menu, machine operation menu options. Touch “Other functions” ,screen display:

Touch linking bag “Open” button, packaging machine operat set parameters to operat continuous automatic packaging. Touch “Close” button, packaging machine stop work.

Return to the main menu, machine operation menu options.Touch “Combination

| Combination weigher feeding one Combination weigher continuous feeding Open Close |  |
| --- | --- |
| Production operation Debuging operation Combination weigher Cup measuring device |  |
| Auger filler Parameter Setting Alarm Settings | Back to homepage |

Weigher”, screen display:

Return to the main menu, machine operation menu options.Touch “Cup measuring device”, screen display:

| Cup measuring device continuous operation Open Close |  |
| --- | --- |
| Production operation Debuging operation Combination weigher Cup measuring device |  |
| Auger filler Parameter Setting Alarm Settings | Back to homepage |

Return to the main menu, machine operation menu options.Touch “Auger filler”

screen display:

| Auger filler feeding one time Auger filler continuous operation Open Close |  |
| --- | --- |
| Production operation Debuging operation Combination weigher Cup measuring device |  |
| Parameter Setting Alarm Settings | Back to homepage |

Return to the main menu, machine operation menu options. Touch “Parameter Settings”,

screen display:

| Bag length setting mm Packing speed |  |
| --- | --- |
| Horizontal delay s Horizontal sealing time s Vertical delay s Vertical sealing time s Cutter delay s Cutter time s Inflatable delay s Inflatable time s Feeding delay s Servo frequency HZ Save parameters |  |
| Production operation Debuging operation Combination weigher Cup measuring device |  |
| Auger fille Alarm Settings | Back to homepage |

Cutter delay XXX means after transfer film to the cutting knife solenoid valves

to the waiting electric time (Units:second)

Cutter XXX means the time cutter knife solenoid valves have electric need.(Units:second)

The set of cutters can cut packaging film, do not affect next bag of falling

Horizontal seal XXX means the time horizontal seal solenoid valves have electric need.(Units:second)

According to the packaging film thickness and packaging speed, packaging film sealing strong not leak.

Vertical seal XXX means the time vertical seal solenoid valves have electric need . (Units:second)

According to the packaging film thickness and packaging speed, packaging film sealing strong not leak.

Bag length XXX means when not to use color code electric eye, the packing film can transfer length(Units:mm)

Inflatable time XXX means the time inflatable solenoid valves have electric need .

Inflatable delay XXX means afterload film before inflatable at this time

Load film XXX means: After process receiving signal to load film before this time

Horizontal seal delayXXX means: before load film to horizontal seal sealing this time.

Vertical seal XXX means: before load film to vertical seal sealing this time.

(Due to the reasons such as skidding belt when load film, the actual length and the packaging value is different from setting value, need to increase to set bag length)

Return to the main menu, machine operation menu options.Touch “Alarm Settings” , screen display:

| Temperature alarm Open Close Low air pressure alarm Open Close Color standard alarm Open Close Delay alarm s No film alarm Open Close Delay alarm s No material alarm Open Close No ribbon alarm Open Close Delay alarm s The security alarm Open Close Save parameters |  |
| --- | --- |
| Production operation Debuging operation Combination weigher Cup measuring device |  |
| Auger fille Parameter Settings | Back to homepage |

When parameters adjustment,press “Save parameters” to certain.

6. Back to homepage, touch “feed back”, screen display:

Touch “open”, begin to feed back material, this time only material turntable rotating, other programs do not move, collecting the materials in the container.Finished, touch “Close” to stop.

Note: Packaging machine parameters Settings, packaging machine can actually have a direct relationship in packing speed. Each parameter values are big, packaging machine speed is slower. Conversely, the more quickly.

Five.   Equipment adjustment

（1）、 Each bag weighs adjust error: By adjusting the fixed materials plate movements to handle move for each bag, can achieve the objective weight error. (pictured)

（2）、 Give material time adjust Give material time not right, appear stuffing , need to adjust the main motor rotor photoelectric, If feeding too eary ,need to make induction plate move along with the direction of rotation, If feeding too late, need to make induction plate move back. (pictured)

（3）Former and vertical heating adjustment(pictured) When former fixed, loosen the handle, shaking adjustment screw, make up with the vertical sealing top and bottom and former give material tube is 2mm apart, lock handle 1.

（4）Vertical sealing bag's adjustment when the edge irregular(pictured) When a, b sides are not neat, on condition that turn adjusting screw loose butterfly screw (first roller can remove from left and right), operation 4,5 bags .observing a, b sides leveled and stable, after locking screws.

(5). Sponge inflatable adjustment Two sponge are the same requirements with density and soft degree , two sponge sealing surface with two horizontal sealing tooth distance required the same, Two sponge sealing surfaces more closer, air filler more big, whereas more smaller.

（6）External air source inflatable adjustment

Open air flow valves, setting up air delay and inflatable time (reference data: all is 0.15)

(7)The vertical sealing flanging adjustment Use hand pulled vertical sealing flanging, packing bag vertical sealing sealing part to designated one side , back into flanging tighten vertical sealing Requirements:A point distance former feeding tube 2mm B point distance former feeding tube 4mm

Six.   Common fault reasons and exclusion

| Fault type | Reasons | The solution |
| --- | --- | --- |
| Touch production operation menu “Open” Machine not work | - Check whether the switch stop reset - Inverter, servo controller display RC | - Reset stop switch - Close power 20s,then open machine |
| Leakage switch trip | Heat electrical leakage | Examine all electrical appliances |
| Sealing roller not hot | - Breaker protection tripping - Heat burn out - Temperature control meter damage | - Excluded the short-circuit,then re-close - Replace heat - Replace temperature control meter |
| Temperature control meter digital repeated beats | Thermocouple loose or damage | Tighten or replace Thermocouple |
| When stop ,temperature is normal . After startup ,temperature decreases | - Environmental is low voltage, cause heat power decrease - Heat power too small | - Reduce packing speed or increase pressure device - Replace |
| Packing seal leakage | - Temperature is not enough - Pressure is not enough - Sealing have not enough time - Front and back roller seal not neat - Stuffing | - Rise temperature proper - Check total pressure is 5.6KG - Add sealing time - Carefully adjusted the parallelism of sealing seal - Adjust feeding material time or inflatable delay |
| Unable to control cutting or cut half bag | - Packaging film edge puckering or light quality problems caused by photoelectric misoperation - Photo-sensor sensitivity not adjustment well - Bag length setting is not enough cause bag not neat | - Replace packing film - Readjustment of photoelectric sensitivity - Enlarge bag length setting |
| Load film not smooth | - Load film cylinder pressure not enough or too big - Belt skid or damage - Packaging film electrostatic too big | - Check load film pressure whether 2.5KG - Use blades shave fur surface or replacement - Replace packing film |

Seven.  Equipment maintenance

1.  Before starting the machine, or close, need to clean the machine, food through parts must strictly disinfected parts.

2. On the day before work, please check the oil tank air three junction are kept in 1/3 above, if less than 1/3, should be injected 20 # pure oil  to about 1/2 of tank.

3. Each day should be at the horizon seal  support  fill 20# oil

4.    When go off work should remove the unused packaging film in order to avoid bending support tube.

5.  If start machine that on the vertical sealing or horizon sealed with material, must immediately stop machine,using copper brush cleaning the vertical sealing or horizon sealed tooth surfaces

Eight.   The circuit principle diagram(one)

Nine The circuit principle diagram(two)

Ten . 220 Pneumatic chart

Eleven.  With machine accessories list

Horizontal heat pipe                    Two pieces

Vertical heat pipe(high-power 400W)                one piece

Vertical heat pipe(low-power 400W)                one piece

Hex wrench  4-10mm                   one units

Load film synchronous belt  560mm                   one piece

Copper brush 120mm                  one piece

Cutting knife   220mm               one piece

Oil pot  150ml              one piece

Word character                       one box

Printer heat pipe              one piece

Date code instructions                  one piece

The thermostat instruction                 one piece

Photo-sensor instruction                one piece

Packaging machine instruction               one piece

Thermocouple                             one piece

Spanner                                  1 piece

Foot (M16)                                4 pieces

Scissors                                  1 piece

Dual approval                                  1 piece

Chromatape                                     1 piece

Handle                                     1 piece

M6 Handle                                  2 pieces

Inflatable mouth + windpipe                1 piece